DECISION RECORD

Adopting a Component-Based Multi-Label Fault Scheme

edge-ai-fault-diagnosis-aerospace  ·  Independent Research Project  ·  Isse Odawa  ·  August 30, 2026

# The Decision

The project's labeling design is changing to match the approach used in Akindoju (2025) — the closest available prior work, using the same MA1DCNN architecture on the same N-CMAPSS dataset, and the direct source of the 98.16% workstation-accuracy baseline this praxis targets. Rather than the earlier plan (three combined files, a single class ID per row, healthy pooled across files), the project now adopts Akindoju's documented approach: all eight N-CMAPSS sub-datasets combined, with genuine multi-label, per-component fault flags derived from a fault-code-to-component mapping table and an RUL-based failure threshold.

# What's Changing

| Aspect | Previous plan | New plan (matches Akindoju, 2025) |
| --- | --- | --- |
| Files used | 3 of 8 (DS01, DS03, DS08a) | All 8 (DS01–DS08c, incl. DS08d — see Open Questions) |
| Label structure | Single class ID per row (0–3) | 5 independent binary flags per row: fan_fail, lpc_fail, hpc_fail, hpt_fail, lpt_fail |
| "Failing" definition | Native hs flag (0/1) | Derived: RUL ≤ 30 cycles marks imminent failure |
| Fault assignment | Whole file → one class ID | Documented per-file component mapping (e.g., DS03 → hpt_fail + lpt_fail) |
| Class imbalance handling | Macro-averaged metrics only | SMOTE oversampling + macro-averaged metrics |
| Approx. engine units | 24 (dev split, 3 files) | ~90–100 (dev+test, 8 files) — restores alignment with the original proposal's "100 units" language |

# Advantages of the New Design

•  Physically meaningful output: a row can correctly show two or more components failing at once (e.g., DS03 → HPT + LPT), matching the praxis's own thesis statement — "identifies which component is failing" — far better than an opaque single class number.

•  Grounded in a documented precedent rather than an independently constructed scheme: the fault-code-to-component mapping and RUL-based threshold come directly from a defended, closely comparable 2025 GWU praxis, not an ad hoc design built without a specific reference point.

•  A more apples-to-apples baseline comparison: since Akindoju's 98.16% figure came from training on all eight files with this exact labeling logic, matching that scope makes this praxis's own workstation baseline directly comparable rather than approximate.

•  Restores alignment with the original proposal's "100 units, 7 failure modes" language — the scope correction made to the documentation earlier this week can now be revised back toward the original framing, this time on solid, verified footing rather than an untested assumption.

•  Enables richer, more standard multi-label evaluation (Hamming loss, Jaccard index, per-label macro-F1) that maps cleanly onto the macro-precision/recall/F1 metrics already specified in the hypotheses.

# Impact on the Praxis

•  H1's accuracy language needs a small clarification: multi-label problems don't have one canonical "accuracy" the way single-label classification does (subset accuracy, per-label accuracy, and Hamming score can all differ). docs/praxis/04-hypotheses.md should specify which one the 93.25% floor applies to.

•  Novelty framing sharpens further: with the data scope and labeling method now matching Akindoju's, the entire point of differentiation is the algorithm-to-hardware work — compression and physical Jetson Nano deployment — which that prior work never attempts. This should be stated plainly wherever the praxis's contribution is described.

•  Data volume increases substantially: five additional files join the three already verified, meaningfully raising total training rows and compute/storage needs for later training and augmentation steps.

•  The earlier "pool healthy across files" reasoning (avoiding shortcut learning) is superseded, not wrong — it was designed for the smaller, file-identity-based scheme. The new per-component flags derived from RUL and a documented mapping don't have the same shortcut-learning risk, since the label no longer depends on which file a row came from in an unexplained way.

# Repository & Code Update Checklist

1. Data Acquisition

•  Check whether the Colab runtime from the last session is still alive (!ls ncmapss_extracted/data_set/data_set/); if not, re-run the wget + both unzip steps.

•  Copy the five remaining files into Drive: N-CMAPSS_DS02-006.h5, DS04.h5, DS05.h5, DS06.h5, DS07.h5, and DS08c-008.h5 (six files, not five — see Open Questions on DS08d).

•  Update notebooks/Uploaded_NCMAPSS_dataset_to_google_drive.ipynb with the additional cp commands and re-push to GitHub.

2. EDA / Verification

•  Repeat the h5py key/shape/unit-count check already run for DS01, DS03, and DS08a against each newly copied file, confirming the same column structure holds across all eight.

•  Decode T_var and X_v_var — still outstanding from the original EDA pass, and now relevant to all eight files rather than three.

•  Update notebooks/01_eda.ipynb with the new cells and re-push.

3. Documentation

•  docs/data-dictionary.md: replace the "Files in use" table with all eight files; replace the "Derived training label" section entirely with the fault-code-to-component mapping table and the RUL ≤ 30 threshold logic, in place of the pooled-healthy 4-class scheme.

•  docs/model-card.md: update "Training data" to reflect 8 files / ~90–100 units / 5-flag multi-label scheme; update the Metrics table to include Hamming loss and Jaccard index alongside macro-precision/recall/F1.

•  docs/praxis/07-data-sources.md: update the dataset description, the illustrative sample-record layout (five fault-flag columns in place of a single hs column), and the H1 hypothesis-alignment row.

•  docs/praxis/04-hypotheses.md: add a one-line clarification of which multi-label accuracy definition the 93.25% floor refers to.

•  docs/praxis/05-annotated-bibliography.md: add the Akindoju (2025) entry already drafted, strengthening the Relevance paragraph to state that this praxis directly adopts its labeling methodology.

4. Future Code (not yet written)

•  src/edge_fault_dx/data/loaders.py: build the fault-code-to-component mapping dictionary and the RUL ≤ 30 threshold logic; load and concatenate all eight files.

•  src/edge_fault_dx/data/preprocessing.py: construct the five-column binary label matrix per row instead of a single class-ID column; normalization, augmentation, and the time-ordered split logic are otherwise unaffected.

•  src/edge_fault_dx/models/ma1dcnn.py: output layer changes from a single softmax over 4 classes to 5 independent sigmoid outputs (standard for multi-label classification), with binary cross-entropy loss per label.

•  src/edge_fault_dx/evaluation/metrics.py: add Hamming loss and Jaccard index alongside the existing macro-precision/recall/F1 plan.

5. Deliverables Folder

•  deliverables/multiclass_fault_classification_plan.docx describes the now-superseded 3-file, single-label design. Recommend keeping it in place as a record of the earlier design iteration rather than deleting it, with this document as the current authoritative decision record.

# Open Questions

The N-CMAPSS download also contains a tenth file, N-CMAPSS_DS08d-010.h5, which does not appear in Akindoju's documented fault-code table (only DS08a and DS08c are listed, both mapped to "all five components / catastrophic"). Before copying DS08d into Drive, confirm what NASA's own documentation (or the bundled N_CMAPSS_Example_data_loading_and_exploration.ipynb) says about it — whether it is a third variant of the same all-component failure mode, a duplicate, or something structurally different — rather than assuming it fits the existing mapping table.